Document No.	:

Title			:	ADALM BNC Adapter Board Test Procedure

| REVISION HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Revision | ECR # | Description of Change | Date | Author |
| A | ECR-102888 | BOM and PSPEC update. Due to different pinout of a new connector there were changes needed to be done in the schematic. Previous ECR was cancelled. | 5/18/2021 | SDeCastr |

| Required Approvers |  |
| --- | --- |
| Approver Roles | Approver Names |
|  | Andreea Pop |

## Overview

## Required hardware

- ADALM2000

- Raspberry Pi + supply, HDMI cable, Screen

- 2 BNC cables

- Test board

## Required setup for production test

- Plug in the Raspberry Pi and connect it to the Screen.

- Plug in the ADALM2000 Into the Raspberry Pi USB port, both LEDS of ADALM2000 should be on

- Connect the channels W1 to 1+ and W2 to 2+ in a loopback configuration, with a BNC cable for each channel

- Connect the AD-M2KBNC-EBZ to the ADALM2000

- Connect the test board to the output header of AD-M2KPWR-EBZ through the 30pin connector, the switch should be in position B

- With a 40-pin ribbon cable, connect the Rpi and the test board. The led on right side of the connector should be on.

Test process

    - After the connections are done as explained in the “Required setup for production test” section double click the file ad-m2kbnc-ebz-test.py

    - The test should start, follow the steps printed in the terminal